# EECS 4421 LAB1

## Name: Mahfuz Rahman

## Student No.: 217847518

### Initial State of robot:

### State of robot after executing code:

# drive_robot.launch.py

import os

from ament_index_python.packages import get_package_share_directory

from launch import LaunchDescription

from launch.actions import DeclareLaunchArgument, IncludeLaunchDescription

from launch.launch_description_sources import PythonLaunchDescriptionSource

from launch.substitutions import LaunchConfiguration

from launch_ros.actions import Node

def generate_launch_description():

return LaunchDescription([

DeclareLaunchArgument('goal_x', default_value = '0.0', description = 'goal (x)'),

DeclareLaunchArgument('goal_y', default_value = '0.0', description = 'goal (y)'),

DeclareLaunchArgument('goal_t', default_value = '0.0', description = 'goal (t)'),

#Code added by Mahfuz Rahman

DeclareLaunchArgument('max_vel', default_value = '0.2', description = 'max_vel (vel)'),

DeclareLaunchArgument('vel_gain', default_value = '5.0', description = 'vel_gain (gain)'),

Node(

package = 'cpmr_ch2',

executable = 'drive_to_goal',

name = 'drive_to_goal',

parameters = [

{'goal_x' : LaunchConfiguration('goal_x')},

{'goal_y' : LaunchConfiguration('goal_y')},

{'goal_t' : LaunchConfiguration('goal_t')},

],

),

])

# drive_to_goal.py

import math

import numpy as np

import rclpy

from rclpy.node import Node

from rclpy.parameter import Parameter

from rcl_interfaces.msg import SetParametersResult

from nav_msgs.msg import Odometry

from geometry_msgs.msg import Twist, Pose, Point, Quaternion

from nav_msgs.msg import Odometry

def euler_from_quaternion(quaternion):

"""

Converts quaternion (w in last place) to euler roll, pitch, yaw

quaternion = [x, y, z, w]

"""

x = quaternion.x

y = quaternion.y

z = quaternion.z

w = quaternion.w

sinr_cosp = 2 * (w * x + y * z)

cosr_cosp = 1 - 2 * (x * x + y * y)

roll = np.arctan2(sinr_cosp, cosr_cosp)

sinp = 2 * (w * y - z * x)

pitch = np.arcsin(sinp)

siny_cosp = 2 * (w * z + x * y)

cosy_cosp = 1 - 2 * (y * y + z * z)

yaw = np.arctan2(siny_cosp, cosy_cosp)

return roll, pitch, yaw

class MoveToGoal(Node):

def __init__(self):

super().__init__('move_robot_to_goal')

self.get_logger().info(f'{self.get_name()} created')

self.declare_parameter('goal_x', 0.0)

self._goal_x = self.get_parameter('goal_x').get_parameter_value().double_value

self.declare_parameter('goal_y', 0.0)

self._goal_y = self.get_parameter('goal_y').get_parameter_value().double_value

self.declare_parameter('goal_t', 0.0)

self._goal_t = self.get_parameter('goal_t').get_parameter_value().double_value

self.add_on_set_parameters_callback(self.parameter_callback)

self.get_logger().info(f"initial goal {self._goal_x} {self._goal_y} {self._goal_t}")

#Code added by Mahfuz Rahman

self.declare_parameter('max_vel', 0.2)

self.declare_parameter('vel_gain', 5.0)

self._max_vel = self.get_parameter('max_vel').get_parameter_value().double_value

self._vel_gain = self.get_parameter('vel_gain').get_parameter_value().double_value

########################

self._subscriber = self.create_subscription(Odometry, "/odom", self._listener_callback, 1)

self._publisher = self.create_publisher(Twist, "/cmd_vel", 1)

def _listener_callback(self, msg, max_pos_err=0.05):

pose = msg.pose.pose

cur_x = pose.position.x

cur_y = pose.position.y

o = pose.orientation

roll, pitchc, yaw = euler_from_quaternion(o)

cur_t = yaw

x_diff = self._goal_x - cur_x

y_diff = self._goal_y - cur_y

dist = math.sqrt(x_diff * x_diff + y_diff * y_diff)

twist = Twist()

if dist > max_pos_err:

x = max(min(x_diff * self._vel_gain, self._max_vel), -self._max_vel)

y = max(min(y_diff * self._vel_gain, self._max_vel), -self._max_vel)

twist.linear.x = x * math.cos(cur_t) + y * math.sin(cur_t)

twist.linear.y = -x * math.sin(cur_t) + y * math.cos(cur_t)

self.get_logger().info(f"at ({cur_x},{cur_y},{cur_t}) goal ({self._goal_x},{self._goal_y},{self._goal_t})")

self._publisher.publish(twist)

def parameter_callback(self, params):

self.get_logger().info(f'move_robot_to_goal parameter callback {params}')

for param in params:

self.get_logger().info(f'move_robot_to_goal processing {param.name}')

if param.name == 'goal_x' and param.type_ == Parameter.Type.DOUBLE:

self._goal_x = param.value

elif param.name == 'goal_y' and param.type_ == Parameter.Type.DOUBLE:

self._goal_y = param.value

elif param.name == 'goal_t' and param.type_ == Parameter.Type.DOUBLE:

self._goal_t = param.value

########

elif param.name == 'max_vel' and param.type_ == Parameter.Type.DOUBLE:

self._max_vel = param.value

elif param.name == 'vel_gain' and param.type_ == Parameter.Type.DOUBLE:

self._vel_gain = param.value

#######

else:

self.get_logger().warn(f'{self.get_name()} Invalid parameter {param.name}')

return SetParametersResult(successful=False)

self.get_logger().warn(f"Changing goal {self._goal_x} {self._goal_y} {self._goal_t}")

return SetParametersResult(successful=True)

def main(args=None):

rclpy.init(args=args)

node = MoveToGoal()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

rclpy.shutdown()

if __name__ == '__main__':

main()